Falls are the leading cause of fatalities in construction, accounting for about one-third of fatalities in the industry. Incidents involving falls are generally complex events, frequently involving a variety of factors. Consequently, the standard for fall protection deals with both the human and equipment-related issues in protecting workers from fall hazards. Use the questions below to gather information during an inspection involving falls on a construction site. 29 CFR 1926, Subpart M

General Information - What phase of work is taking place on the site? - What is the multi-employer structure? - What activities are taking place on site? - Who is the Competent Person on site? - Is the designated Competent Person competent? - Where are employees subject to fall hazards at or above 6 feet? - Are workers subject to falls into impalement hazards, dangerous equipment or sources of uncontrolled energy? - Are workers trained in to recognize a properly built guardrail system or other fall protection systems? - Are workers trained in how to wear full-body harnesses and use a personal fall arrest system (PFAS)? - Do guardrail systems currently exit on site? - Do guardrail systems meet the standard criteria? - Are workers currently wearing PFAS? - Rescue plan in place? - If PFASs are currently in use, are they being worn and used properly? In good condition? - Are the workers tied off? - Are the PFAS anchors appropriate? - Does the company have a Fall Protection Plan? - Is housekeeping maintained through all phases of work? - Do all workers have certified fall protection training? - Has anyone ever fallen on the site? - Have there been any near misses on the site? - Are there wall openings and/or floor holes on the site? - Do any guardrail systems exist on site? - Are wall openings and floor holes appropriately guarded and covered? - Are floor hole covers adequately marked and secured? - Do ladders and stairways exist on the site? Ladder stickers present and legible? - Are job built stairways and ladders being used on site? - Who uses the stairways and ladders? - Who inspects the stairways and ladders? - Are stairways and ladders appropriate, adequate and in good condition? - Are employees trained on safe ladder use? - Are scaffolds being used on site? - What type of scaffold(s) is being used on site? - How high is the scaffold? - Is the scaffold fully planked, properly level, and constructed of appropriate materials? - Is excavation and trenching work taking place on site? - Are there open excavations/trenches on site? - Are appropriate warning systems in place to prevent falls into excavations/trenches?