Moving machine parts have the potential to cause severe workplace injuries, such as crushing fingers or hands, amputations, burns, or blindness. Safeguards are essential for protecting workers from these preventable injuries. Any machine part, function, or process that may cause injury must be safeguarded. When the operation of a machine or accidental contact injure the operator or others in the vicinity, the hazards must be eliminated or controlled. Used the questions below as appropriate to gather information during an inspection involving machine guarding. 29 CFR 1910, Subpart O, 29 CFR 1926, Subpart I

Requirements for All Safeguards - Do the safeguards provide meet the minimum OSHA requirements? - Do the safeguards prevent workers’ hands, arms, and other body parts from making contact with dangerous moving parts? - Are the safeguards firmly secured and not easily removable? - Do the safeguards ensure that no object will fall into the moving parts? - Do the safeguards permit safe, comfortable, and relatively easy operation of the machine? - Can the machine be oiled without removing the safeguard? - Is there a system for shutting down the machinery and locking/tagging out before safeguards are removed? - Can the existing safeguards be improved? - Are employees working within 7 feet of the hazard? - Any obvious handmade alterations to machine or tools? Machine Hazards - Is there a point-of-operation safeguard provided for the machine? - Does it keep the operator’s hands, fingers, body out of the danger area? - Is there evidence that the safeguards have been tampered with or removed? - Could changes be made on the machine to eliminate the point-of-operation hazard entirely? - Are machines locked out for maintenance procedures? Power Transmission Apparatus - Are there any unguarded gears, sprockets, pulleys, or flywheels or the apparatus? - Are there any exposed belts or chain drives? - Are there any exposed set screws, key ways, collars, etc.? - Are starting and stopping controls within easy reach of the operator - If there is more than one operator, are separate controls provided? Other Moving Parts - Are safeguards provided for all hazardous moving parts of the machine, including auxiliary parts? Non-Mechanical Hazards - Have appropriate measured been taken to safeguarded workers against noise hazards? - Have special guards, enclosures, or personal protective equipment been provided, where necessary to protect workers from exposure to harm substances used in machine operation? Electrical Hazards - Is the machine installed in accordance with National Fire Protection Association and National Electrical Code requirements? - Are there loose conduit fittings - Is the machine properly grounded? - Is the power supply correctly fused and protected? - Do workers occasionally receive minor shocks while operating any of the machine?

What type of guards are on the machine? - Fixed Guards - Interlocks - Adjustable - Curtains - Pull backs - Barrier - Two handed tripping device - Self-Adjusting

What is being guarded? - Chains - Sprockets - Pulleys - Blades - Shafting - Conveyor Belt - Fly wheels - Key ways/Set screws - Cutting blades - Point of Operation

What is the hazard? - Pinch - Point of Operation - Rotating - Reciprocating - Cutting - Caught in - Struck - Rotating parts - Compaction

What body part is affected? - Hands - Arms - Knees - Head - Eyes